| Author | Cailen Matz | Approver(s) | Peter Langel, Jayson Fajardo |
| --- | --- | --- | --- |

Revision Log:

| REV | Revision Description (Reason for Change) | Author | Rev. Date |
| --- | --- | --- | --- |
| 1.0 | Initial Release | C.Matz | 2/24/2022 |
|  |  |  | DRAFT 2-24 |

# LABOR CODE:???

PURPOSE: This tool is part of a kit used at collision centers to lift the R1T body off the skateboard. Adapters attach at the gear tunnel hinge bolt locations.

GENERAL PRINCIPLES:

  - Follow safe vehicle lifting practices.  Always check the stability of the vehicle before lifting.

  - Do not use the lift adapter for any vehicle other than R1T.

  - Do not perform lift alone, at minimum 1 spotter on the driver’s side and one on the passenger’s side required.

  - Check the lift adapters for any damage, cracks, etc.  If the adapters are not in proper working order, discontinue use and get a new set.

TOOL BREAKDOWN:

R1 RR ADAPTER (x2)

M8 x 1.25 Flange Screw (x8)

DRAFT 2-24

TOOL LOCATION:  DRIVERS SIDE

FWD

TOOL LOCATION:  PASSENGERS SIDE

FWD

USING THE TOOL:

- Align adapter to gear tunnel hinge mount studs (4x) on the passenger’s side of the vehicle.  Hand tighten M8 x 1.25 fasteners.

DRAFT 2-24

- Torque fasteners to 38 Nm

- Repeat steps 1 and 2 for driver’s side.

- Raise the lift and position just below the adapters.  Adjust lift pad heights to ensure they contact both adapters simultaneously.