| Author | Lee Esten | Approver(s) |  |
| --- | --- | --- | --- |

Revision Log:

| REV | Revision Description (Reason for Change) | Author | Rev. Date |
| --- | --- | --- | --- |
| 1 | Initial Doc, no revision | L.E. |  |

Reference

Labor Code 522299427 Bolted Body Components Other Labor (Multiple labor codes may apply)

PURPOSE: The purpose of this tool is to clean up nut/bolt fastener threads that contain thread-lock.

GENERAL PRINCIPLES

  - Refer to Service Manual for directions in reusing or replacing fasteners that contain thread locker.

  - Always discard and replace damaged fasteners.

  - Do not use these tools to attempt to repair damaged threads on fasteners.

  - Pay careful attention to match correct thread pitch on top of thread size.

  - If the incorrect tool is used, and causes damage to the fastener, replace the fastener.

USING THE TOOL

- Only use this tap and die set to clean up nut/bolt fasteners that are filled with dried up thread-lock.

- Measure the size of the bolt and or nut as well as thread pitch.

- If thread pitch information is unavailable, slowly, and carefully try a thread chaser, if you feel resistance stop.

- Note that for some thread chaser sizes, there are multiple thread pitches, ensure you are using the correct size and pitch.

- Apply new thread lock if required. Check the service manual to ensure the proper thread locker is applied.